# FTR Schematic Checklist

# First-Time-Right Schematic Checklist

Design: AtmosTag
Revision: V1.2
MCU: STM32L031G6U6
EPD: 2.13" e-paper
Primary analog: GUVA-S12SD + MCP6002
USB: CP2102N
EDA: EasyEDA

# 1. Requirements Before Schematic Freeze

- Every electrical requirement has a measurable acceptance criterion.

- Input voltage range has been defined, including minimum, nominal, and maximum.

- Required operating temperature range has been defined.

- Maximum active current has been estimated.

- Maximum sleep current has been estimated.

- Battery/source characteristics have been defined.

- Required EPD operating voltages have been taken from the actual EPD specification.

- Required EPD refresh behavior has been defined.

- Required UV measurement range and accuracy have been defined.

- Required temperature/humidity measurement accuracy has been defined.

- USB operating requirements have been defined.

- Required MCU clock accuracy has been defined.

- Every requirement has been traced to at least one schematic design decision.

# 2. Datasheet-to-Schematic Verification

- The latest datasheet for every critical IC has been reviewed.

- Recommended application circuit has been compared against the schematic.

- Absolute maximum ratings have been checked.

- Recommended operating conditions have been checked.

- Minimum/maximum external component values have been checked.

- Required external components have been identified rather than assuming the reference circuit is complete.

- Device-specific startup requirements have been checked.

- Shutdown/low-power requirements have been checked.

- Internal pull-ups/pull-downs have been identified before adding external ones.

- Reserved and NC pins have been handled according to the datasheet.

- Package-specific pinout has been checked rather than relying on a generic symbol.

- Datasheet assumptions about external circuitry have been explicitly verified.

### Critical devices for independent verification

- STM32L031G6U6

- SHT41-P-NF

- MCP6002-I/MS

- GUVA-S12SD

- CP2102N-A02-GQFN20

- SI1304BDL

- AO3401A

- MBR0530

- EPD panel/interface specification

- EPD boost/reference circuit

# 3. MCU Pin Assignment

- Every MCU pin assignment has been checked against the STM32 alternate-function table

- No peripheral function conflicts with another required peripheral.

- Analog pins are actually ADC-capable where required.

- I²C pins support the intended I²C peripheral/function.

- UART pins support the intended UART peripheral/function.

- SWD pins remain available.

- Crystal pins support the intended 32.768 kHz oscillator.

- Boot configuration does not conflict with normal GPIO use.

- Every GPIO's power-up/reset state has been checked.

- Every GPIO connected to an external circuit has been checked for back-powering risk.

- Every GPIO connected to an external circuit has been checked for maximum source/sink current.

- No GPIO is relied upon to remain at a specific state before firmware initialization unless hardware guarantees it.

- Pin assignment has been checked against the final firmware pin definitions.

# 4. Power Tree

- A complete power-tree diagram has been derived from the schematic.

- Every power rail has a defined source, nominal voltage, tolerance, and maximum current.

- Worst-case load has been calculated for every rail.

- Startup load has been considered separately from steady-state load.

- Peak/transient load has been considered separately from average load.

- Power sequencing has been checked for every IC that has sequencing requirements.

- Power-down sequencing has also been checked where relevant.

- No rail can unintentionally be powered through an I/O pin.

- No powered-down IC can back-power another rail.

- USB VBUS and board power paths have been checked for backfeeding.

- Analog and digital power domains are isolated

- All rail tolerances have been propagated to downstream devices.

# 5. Power Budget & Low-Power Design

- Active current budget has been calculated from individual loads.

- Sleep current budget has been calculated from individual loads.

- Worst-case resistor leakage/current has been included.

- Pull-up resistor current has been included.

- LED current has been included.

- SHT41 consumption has been included.

- MCP6002 quiescent current has been included.

- CP2102N standby/suspend consumption has been included.

- EPD boost quiescent/leakage current has been included.

- EPD boost is guaranteed to be disabled when not required.

- UV circuitry can be disabled when not measuring.

- Analog circuitry does not remain unintentionally powered during sleep.

- Maximum sleep current remains below the system requirement with component tolerances.

- Worst-case battery/source voltage has been used in the power analysis.

- Worst-case temperature effects on current consumption have been considered where important.

# 6. Power Protection & Fault Analysis

- Reverse-polarity protection has been analyzed for the actual expected fault.

- Overvoltage behavior has been analyzed.

- Short-circuit behavior has been analyzed.

- USB power insertion/removal has been analyzed.

- Simultaneous USB and external power has been analyzed.

- Backfeed paths have been traced manually.

- Every externally accessible signal has been checked against MCU/IC absolute maximum ratings.

- ESD protection requirements have been determined from the actual external interfaces.

- Protection components have been selected based on worst-case fault voltage/current

- Protection components do not interfere with normal signal operation.

- Failure of a protection component does not create an unsafe power path.

# 7. Decoupling

- Decoupling requirements have been obtained from each IC datasheet.

- Required capacitance has been checked against actual selected values.

- Capacitor DC-bias derating has been considered.

- Capacitor tolerance has been considered.

- Capacitor voltage rating has adequate margin.

- Required bulk capacitance has been calculated from load transients where necessary.

- Regulator stability requirements are satisfied with the selected capacitor type/value.

- No regulator depends on a capacitor whose effective capacitance falls outside its stability range.

- High-current switching sections have been checked for adequate local energy storage.

- Analog/reference rails have sufficient filtering for the required measurement accuracy.

# 8. STM32L0 Reset, Boot & Startup

- NRST circuit has been checked against the STM32 reset requirements.

- NRST pull-up/pull-down value is appropriate.

- Reset capacitor, if used, has been checked against required reset timing.

- BOOT0 default state has been verified.

- Boot configuration remains valid during power ramp-up.

- SWD remains usable under all intended power conditions.

- No external peripheral can force an MCU pin into an invalid state during reset.

- EPD_EN, UV_EN, and other power-control signals have defined hardware states before firmware starts.

- EPD cannot accidentally start during MCU reset.

- UV circuitry cannot accidentally activate during MCU reset.

- Startup behavior has been considered before GPIO initialization.

# 9. 32.768 kHz Oscillator

- Crystal load capacitance has been calculated

- MCU pin capacitance and PCB parasitics have been included in the calculation.

- Selected capacitor values provide the required effective load capacitance.

- Crystal ESR is compatible with the STM32 oscillator.

- Crystal drive level is acceptable.

- Startup time is acceptable for the intended low-power operation.

- Oscillator accuracy meets the system requirement.

- Temperature tolerance of the crystal has been considered

- Crystal operation has been checked against the STM32 oscillator configuration.

# 10. SHT41 I²C Interface

- I²C pull-up values have been calculated from bus capacitance and required speed.

- Minimum and maximum pull-up current are acceptable.

- MCU and SHT41 logic thresholds are compatible across the full voltage range.

- SDA/SCL behavior during reset has been checked.

- No device can hold the bus in an unintended state during startup.

- Bus recovery requirements have been considered.

- Sensor supply tolerance is within specification.

- Sensor measurement timing is compatible with the firmware timing requirement.

- Sensor self-heating implications have been considered.

- SHT41 low-power operating mode has been included in the power budget.

# 11. UV Sensor / TIA

## Photodiode

- GUVA-S12SD spectral response is appropriate for the intended measurement.

- Photodiode polarity has been independently verified.

- Photodiode operating/bias configuration is intentional.

- Photodiode capacitance has been obtained from the datasheet

- Maximum and minimum expected photocurrent has been established.

TIA

- TIA transimpedance gain has been calculated from the required input range.

- Maximum photodiode current does not saturate the amplifier.

- Minimum photodiode current produces sufficient ADC resolution.

MCP6002 input common-mode range is satisfied.

- MCP6002 output swing is sufficient at both minimum and maximum signal.

- Feedback resistor tolerance has been included in gain accuracy.

- Feedback capacitor has been selected from a stability/bandwidth analysis.

- Photodiode capacitance and amplifier input capacitance have been included in the stability analysis.

- TIA phase margin/stability has been considered.

- TIA bandwidth is sufficient for the intended measurement.

- TIA bandwidth is not unnecessarily large for a low-noise sensor measurement.

- Op-amp input bias current error has been checked against the minimum signal.

- Op-amp input offset error has been checked against the required measurement accuracy.

- Resistor thermal noise has been considered where it materially affects the measurement.

## Second Amplifier / ADC

- Second-stage gain has been calculated.

- Any intended offset/bias voltage has been calculated.

- Maximum UV_SIG remains below the STM32 ADC maximum input voltage.

- Minimum UV_SIG remains inside the intended ADC operating range.

- ADC resolution is sufficient for the required UV measurement resolution.

- ADC source impedance is compatible with STM32 ADC acquisition requirements.

- RC filtering has been checked against ADC sampling/acquisition time.

- Total analog gain has been checked from photodiode current → TIA → amplifier → ADC.

- Worst-case component tolerances have been propagated through the complete analog chain.

- Saturation recovery behavior has been considered.

# 12. Analog Power & Ground Architecture

- AVCC filtering has been justified by the analog accuracy requirement.

- MCP_AVCC voltage remains valid under worst-case analog load.

- Analog supply ripple requirement has been established.

- AGND/GND connection strategy is deliberate.

- High-current EPD switching currents have a defined return path that does not compromise the analog reference.

- Digital switching noise coupling into the UV measurement has been considered at schematic level.

- ADC reference/supply noise has been considered.

- Analog measurement accuracy includes supply and ground error sources.

# 13. EPD Interface

- The actual EPD panel datasheet/interface specification has been used.

- Every FPC pin has been mapped from panel specification → connector → schematic net.

- Connector pin numbering has been checked independently from the symbol.

- Connector orientation has been checked independently from the footprint.

- Logic supply voltage is compatible with the EPD.

- EPD logic signal voltage levels are compatible with STM32.

- EPD_BUSY direction is correct.

- EPD_RST polarity is correct.

- EPD_DC polarity/function is correct.

- EPD_CS polarity/function is correct.

- EPD_BS configuration is correct.

- EPD_RESE/GDR functions have been verified against the actual driver architecture.

- No panel pin has been inferred from another panel's pinout.

- No unused panel pin has been assumed to be harmless without checking the panel specification.

# 14. EPD High-Voltage Boost

- The exact EPD required VGH/PREVGH voltage has been verified.

- The exact EPD required VGL/PREVGL voltage has been verified.

- Positive/negative rail polarity has been independently verified.

- Required voltage tolerance has been established.

- Maximum EPD load on each HV rail has been established.

- Boost topology has been verified against the intended EPD driver/reference circuit.

- Inductor value has been verified from the converter design requirements.

- Inductor saturation current exceeds worst-case peak current with margin.

- Inductor RMS current rating is adequate.

- Inductor DCR loss has been considered.

- Diode reverse-voltage rating exceeds the worst-case node voltage.

- Diode peak and average current ratings are adequate.

- MOSFET VDS rating has adequate margin over every possible switching-node voltage.

- MOSFET VGS rating is respected during startup and shutdown.

- MOSFET RDS(on) is acceptable at the actual gate voltage.

- Switching losses have been considered.

- Conduction losses have been considered.

- Boost efficiency has been estimated.

- Worst-case thermal dissipation has been estimated.

- HV capacitor voltage ratings include sufficient margin.

- Effective capacitance after DC-bias derating is sufficient.

- HV capacitor ripple current is acceptable.

- PREVGH and PREVGL loads have been analyzed separately.

- HV startup time is acceptable.

- HV discharge behavior after disable has been considered.

- EPD_EN cannot produce an undefined boost state.

- Boost startup does not violate any EPD sequencing requirement.

- Boost shutdown does not leave an invalid EPD state.

- No HV node can reach an MCU GPIO under a single-component fault that is reasonably foreseeable.

- The complete boost circuit has been compared against the actual reference design, not merely judged from topology appearance.

# 15. USB / CP2102N

- CP2102N power architecture has been checked against the datasheet/reference design.

- VBUS/VREGIN/VDD relationships are correct.

- USB D+ and D− connections are correct.

- USB logic voltage is compatible with STM32.

- CP2102N TX/RX directions are correct.

- CP2102N reset behavior has been checked.

- Required CP2102N decoupling is present and correctly specified.

- USB ESD protection voltage rating is appropriate.

- USB ESD protection capacitance is acceptable.

- USB VBUS loading is within specification.

- USB suspend/low-power behavior has been considered.

- USB cannot unintentionally power the rest of the system through signal pins.

- Simultaneous USB and other power-source conditions have been analyzed.

- USB interface behavior during MCU reset has been considered.

# 16. MOSFET-Controlled Power Sections

For Q1/Q2/Q3/Q4:

- MOSFET topology has been identified: high-side, low-side, load switch, reverse protection, etc.

- Body-diode direction has been explicitly checked.

- Gate default state has been explicitly defined.

- Gate voltage during MCU reset has been checked.

- Gate-source voltage remains within limits.

- MOSFET is fully enhanced at the actual available gate voltage.

- RDS(on) has been checked at that gate voltage.

- Maximum current has been calculated.

- Power dissipation has been calculated.

- Worst-case junction temperature has been estimated.

- Turn-on/turn-off behavior is appropriate for the connected load.

- No MOSFET can unintentionally latch a power rail on.

- No MOSFET body diode creates an unwanted backfeed path.

# 17. Passive Component Worst-Case Analysis

- Every resistor with a functional role has a tolerance requirement.

- Every critical resistor has been checked for power dissipation.

- Every critical capacitor has been checked for voltage derating.

- Every timing/filter capacitor has been checked for tolerance.

- Every inductor has been checked for saturation and RMS current.

- Every diode has been checked for worst-case voltage and current.

- Every MOSFET has been checked for voltage, current, gate voltage, and power.

- Component tolerances cannot push an ADC input outside its valid range.

- Component tolerances cannot cause analog-stage saturation.

- Component tolerances cannot cause an EPD rail to exceed its permitted voltage.

# 18. Startup, Shutdown & State-Transition Analysis

Review the schematic at these exact states:

### Power applied

- All rails rise to valid levels.

- MCU starts in a defined state.

- EPD boost remains disabled until intentionally enabled.

- UV circuit remains disabled until intentionally enabled.

- No peripheral back-powers another rail.

### MCU reset

- Every externally controlled signal has a safe state.

- EPD does not receive an invalid command/control sequence.

- EPD HV generation remains safe.

- UV circuitry remains safe.

- Buttons do not create unintended current paths.

### MCU sleep

- Intended power domains actually become inactive.

- No GPIO back-powers a peripheral.

- Pull-up/pull-down currents are acceptable.

- USB circuitry behaves as intended.

- EPD HV rails behave as intended.

### Wake-up

- Required peripherals can be powered/enabled correctly.

- I²C bus starts in a valid state.

- Analog circuitry reaches a settled state before ADC measurement.

- EPD supply reaches valid conditions before EPD commands are issued.

### Power removal

- No rail is held up unintentionally.

- No IC is partially powered through an I/O.

- EPD HV rails discharge appropriately.

- No reverse-current path exists.

# 19. ERC & EasyEDA Integrity

- ERC has been run after the schematic is functionally complete.

- Every ERC error has been investigated.

- Every ERC warning has been investigated.

- ERC exceptions are intentional rather than ignored for convenience.

- Power-input pins are correctly recognized by EasyEDA.

- Intentional power connections are explicitly recognized by the ERC system.

- No hidden global-net connection creates an unintended circuit.

- No duplicate net label creates an unintended connection.

- No symbol pin type incorrectly suppresses a real electrical error.

- ERC-clean status has not been treated as proof of functional correctness.

# 20. Independent Review

Before schematic freeze:

- A second pass has been performed without looking at the previous checklist results.

- Critical IC pinouts have been independently rechecked.

- EPD FPC mapping has been independently rechecked.

- EPD HV polarity and voltage requirements have been independently rechecked.

- UV TIA calculations have been independently rechecked.

- Power-tree calculations have been independently rechecked.

- USB/CP2102N power and signal architecture has been independently rechecked.

- Worst-case component ratings have been independently rechecked.

- Startup/shutdown behavior has been independently reviewed.

- Firmware-dependent assumptions have been explicitly identified.

- Any remaining uncertainty has been resolved from a datasheet, reference design, calculation, or experiment.

# 21. Schematic Freeze Gate

Do not freeze the schematic until:

- All critical calculations have numerical results.

- All critical IC circuits have been compared against their reference designs.

- All critical interfaces have been independently pin-checked.

- All power rails have a completed worst-case budget.

- All analog gains/ranges have a completed worst-case calculation.

- EPD HV generation has a completed worst-case voltage/current/thermal analysis.

- Startup and shutdown behavior has been reviewed.

- Fault/backfeed paths have been reviewed.

- ERC is clean or every exception is documented.

- BOM part numbers have been frozen for all electrically critical components.

- No component value is selected merely because it is a "typical" value.

- No circuit block is accepted merely because it resembles a reference schematic.

- Every unresolved design assumption has been eliminated or explicitly accepted as a known risk.

## First-Time-Right Critical Gates

For this board, these are the highest-priority schematic checks:

- EPD FPC pin mapping independently verified

- EPD VGH/VGL requirements independently verified

- EPD boost topology mathematically verified

- EPD MOSFET/diode/inductor worst-case ratings verified

- EPD startup/shutdown sequence verified

- GUVA-S12SD polarity and operating configuration verified

- 1 MΩ TIA gain and stability verified

- Complete UV analog gain/range calculated

- UV_SIG worst-case ADC range verified

- MCP6002 input/output limitations verified

- VCC → AVCC → MCP_AVCC power architecture verified

- USB/CP2102N power architecture verified

- All USB/external-power backfeed paths verified

- STM32 boot/reset GPIO states verified

- Sleep-current budget calculated

- All critical component tolerances propagated

- Startup/shutdown/fault states reviewed

- Independent second review completed

## Sign-Off

Designer: ____________________________

Reviewer: ____________________________

Revision: ____________________________

Date: ________________________________

ERC: Pass / Exceptions documented / Fail

Power analysis: Pass / Fail

Analog analysis: Pass / Fail

EPD HV analysis: Pass / Fail

Interface verification: Pass / Fail

Startup/fault analysis: Pass / Fail

Independent review: Pass / Fail

Schematic frozen: Yes / No

# Schematic Data

## MCU Block

### Datasheet Informations

- U1: STM32L031G6U6

Supply Voltage	: 1.65V ~ 3.6V

Temperature		: -40°C ~ 80°C

Package		: UFQFPN-28

- X1: Seiko SC32S
Frequency		: 32.768Hz

Load Capacitance	: 6pF ~ 12pF

### Capacitor & Resistor Requirements (AI Generated)

- C2 & C3

Capacitance		: 6pF ~ 12pF

Voltage Rating		: 10V or higher

Tolerance		: +- 5% or lower

Minimum package	: 0402

Type			: C0F or NP0
Placement		: near X1

- C1, C4, C5, C32

Capacitance		: 100nF

Voltage Rating		: 16V or higher

Tolerance		: +- 10%

Minimum package	: 0402

Type			: X7R or X5R

Placement		: Near designated pin

- C33

Capacitance		: 4.7uF
		Voltage Rating		: 10V or higher

Tolerance		: +- 10%

Minimum package	: 0603

Type			: X7R or X5R

Placement		: Near power pin

- R13

Resistance		: 100Ω

Tolerance		: +- 5%

Power Rating		: 1/16W

Minimum package	: 0402

- R17

Resistance		: 10kΩ

Tolerance		: +- 5%

Power Rating		: 1/16W

Minimum package	: 0402

### STM32 Pin Data

Source: Table 15. From STM32L031x4/6 Datasheet

| Pin No. | Pin Name | Pin Type | Alternate/Additilonal Functions | Assigned As |
| --- | --- | --- | --- | --- |
| 1 | VDD | S | - | VCC |
| 2 | PC14 - OSC32 IN | I/O | - | OSC_IN |
| 3 | PC15 - OSC32 OUT | I/O | - | OSC_OUT |
| 4 | NRST | I/O | - | NRST |
| 5 | VDDA | S | - | AVCC |
| 6 | PA0 - CK_IN | I/O | COMP1_INM6, ADC_IN0, RTC_TAMP2/WKUP1, LPTIM1_IN1, TIM2_CH1, USART2_CTS, TIM2_ETR, COMP1_OUT | UV_SIG (ANALOG INPUT: ADC_IN0) |
| 7 | PA1 | I/O | EVENTOUT, LPTIM1_IN2, TIM2_CH2, I2C1_SMBA, USART2_RTS, TIM21_ETR, COMP1_INP, ADC_IN1 | UV_EN (DIGITAL OUTPUT) |
| 8 | PA2 | I/O | TIM21_CH1, TIM2_CH3, USART2_TX, LPUART1_TX, COMP2_OUT, COMP2_INM6, ADC_IN2, RTC_TAMP3/RTC_TS/ RTC_OUT/WKUP3 | STM32_TX (LPUART1_TX) |
| 9 | PA3 | I/O | TIM21_CH2, TIM2_CH4, USART2_RX, LPUART1_RX, COMP2_INP, ADC_IN3 | STM32_RX (LPUART1_RX) |
| 10 | PA4 | I/O | SPI1_NSS, LPTIM1_IN1, USART2_CK, TIM22_ETR COMP1_INM4, COMP2_INM4, ADC_IN4 | EPD_BUSY (DIGITAL INPUT) |
| 11 | PA5 | I/O | SPI1_SCK, LPTIM1_IN2, TIM2_ETR, TIM2_CH1, COMP1_INM5, COMP2_INM5, ADC_IN5 | EPD_CLK (SPI1_SCK) |
| 12 | PA6 | I/O | SPI1_MISO, LPTIM1_ETR, LPUART1_CTS, TIM22_CH1, EVENTOUT, COMP1_OUT, ADC_IN6 | EPD_CS (OUTPUT) |
| 13 | PA7 | I/O | SPI1_MOSI, LPTIM1_OUT, USART2_CTS, TIM22_CH2, EVENTOUT, COMP2_OUT, ADC_IN7 | EPD_DIN (SPI1_MOSI) |
| 14 | PB0 | I/O | EVENTOUT, SPI1_MISO, USART2_RTS, TIM2_CH3 ADC_IN8, VREF_OUT | EPD_DC (OUTPUT) |
| 15 | PB1 | I/O | USART2_CK, SPI1_MOSI, LPUART1_RTS, TIM2_CH4, ADC_IN9, VREF_OUT | EPD_RST (OUTPUT) |
| 16 | VSS | S | - | GND |
| 17 | VDD | S | - | VCC |
| 18 | PA8 | I/O | MCO, LPTIM1_IN1, EVENTOUT, USART2_CK, TIM2_CH1 | EPD_EN (OUTPUT) |
| 19 | PA9 | I/O | MCO, I2C1_SCL, USART2_TX, TIM22_CH1 | I2C_SCL |
| 20 | PA10 | I/O | I2C1_SDA, USART2_RX, TIM22_CH2 | I2C_SDA |
| 21 | PA13 | I/O | SWDIO, LPTIM1_ETR, LPUART1_RX | SWDIO |
| 22 | PA14 | I/O | SWCLK, LPTIM1_OUT, I2C1_SMBA, USART2_TX, LPUART1_TX | SWCLK |
| 23 | PA15 | I/O | SPI1_NSS, TIM2_ETR, EVENTOUT, USART2_RX, TIM2_CH1 | No Connection |
| 24 | PB3 | I/O | SPI1_SCK, TIM2_CH2, EVENTOUT, COMP2_INN | LED (DIGITAL OUTPUT) |
| 25 | PB6 | I/O | USART2_TX, I2C1_SCL, LPTIM1_ETR, TIM21_CH1 COMP2_INP | BTN_2 (DIGITAL INPUT) |
| 26 | PB7 | I/O | USART2_RX, I2C1_SDA, LPTIM1_IN2, COMP2_INP, VREF_PVD_IN | BTN_1 (DIGITAL INPUT) |
| 27 | BOOT0 | I | - | BOOT0 |
| 28 | VSS | S | - | GND |

Note:

Pin Type:

- S: Supply Pin

- I: Input Only

- O: Output Only

- I/O: Input Output

- B: Boot0 Pin

- RST: Bidirectional reset pin

## 2. Power Circuitry

### Electrical Rating Requirements

- Power Source		: CR2032 Battery

  - Voltge		: 3V

  - Internal R	: 15Ω ~ 40Ω

  - Capacity	: 200 mAh ~ 250 mAh

- Absolute Maximum

  - Voltage	: 3.5V

  - Current	: 3mA

### Components Specification Requirements (AI Generated)

- L1 (Ferrite Bead)

Impedance		: 600Ω @ 100 MHz

Current Rating		: 200mA ~ 500mA

Minimum package	: 0603

- C6, C8

Capacitance		: 100nF
		Voltage Rating		: 16V or higher

Tolerance		: +- 10%

Minimum package	: 0402

Type			: X7R or X5R

Placement		: Near ferrite bead

- C23, C7

Capacitance		: 100uF
		Voltage Rating		: 10V or higher

Tolerance		: +- 20%

Minimum package	: A-Case or similar

Type			: Tantalum

Placement		: Near ferrite bead

- R6, R18
Package		: 0605

## 3. Push Button

### Components Specification Requirements (AI Generated)

- SW1, SW2
Minimum cap Ø	: 2mm

Package		: SMD

- C30, C31

Capacitance		: 100nF

Voltage Rating		: 16V or higher

Tolerance		: +- 10%

Minimum package	: 0402

Type			: X7R or X5R

Placement		: Near designated pin

## 4. SHT41

### Datasheet Informations

- SHT41

Supply Voltage	: 1.08V ~ 3.6V

Temperature		: -40°C ~ 125°C

Current		: 80 nA ~ 0.4 uA

Interface		: I2C

I2C Address		: 0x44/0x45/0x46

### Components Specification Requirements (AI Generated)

- R1, R2

Resistance		: 4.7kΩ

Tolerance		: +- 5%

Power Rating		: 1/16W

Minimum package	: 0402

- C10

Capacitance		: 100nF

Voltage Rating		: 16V or higher

Tolerance		: +- 10%

Minimum package	: 0402

Type			: X7R or X5R

Placement		: Near the power pin

## 5. UV Sensor

GUVA-S12SD Simulation Data

### Datasheet Informations

- GUVA-S12SD

Spectral Range	: 240 nm ~ 370 nm

Photo Current		: 21 nA / 1 UVI

Soldering Temp	: 260°C

- MCP6002
Supply Voltage	: 1.8V ~ 6.0V
Operating Temp	: -40°C ~ 85°C
Package		: MSOP-8
Quiescent Current	: 100 uA ~ 320 uA
Soldering Temp	: 260°C

- AO3401A
MOSFET Type	: P-Channel
Quiescent Current	: 0.1uA ~ 1uA

### Components Specification Requirements (AI Generated)

- C9

Capacitance		: 100nF
		Voltage Rating		: 16V or higher

Tolerance		: +- 10%

Minimum package	: 0402

Type			: X7R or X5R

Placement		: Near photodiode

- R3

Resistance		: 1M Ω

Tolerance		: +- 1%

Power Rating		: 1/16W

Minimum package	: 0402

- R8

Resistance		: 470k Ω

Tolerance		: +- 5%

Power Rating		: 1/16W

Minimum package	: 0402

- Post-TIA Level Shifting

  - R4

Resistance		: 3.3k Ω

Tolerance		: +- 1%

Power Rating		: 1/16W

Minimum package	: 0402

- R5

Resistance		: 1k Ω

Tolerance		: +- 1%

Power Rating		: 1/16W

Minimum package	: 0402

- Op-Amp Gain

  - R6

Resistance		: 1k Ω

Tolerance		: +- 1%

Power Rating		: 1/16W

Minimum package	: 0402

- R7

Resistance		: 2.2k Ω

Tolerance		: +- 1%

Power Rating		: 1/16W

Minimum package	: 0402

### Op-Amp Configuration Requirements:

Maximum output voltage		: 1.2V (STM32 VREFINT)

TIA					: 21 nA -> 21mV

Op-Amp Gain				: 3.2x: 21mV -> 0.07V

GUVA-S12SD Simulation Data - Google Sheets

## 6. USB Interface

Typical Application (Source: CP2102N Datasheet)

### Datasheet Informations (CP2102N)

Part Number		: CP2102N-A02-GQFN20R

Supply Volt (VREGIN): 3.6V to 5.25V (from USB VBUS)

Regulated VOUT	: 3.3V (Internal Regulator Output)
Operating Temp	: -40°C ~ 85°C

Comm. Interface	: USB 2.0 Full-Speed (12 Mbps) to UART Bridge

### Components Specification Requirements (AI Generated)

- USB1
Conn. Type	: Micro-B USB Receptacle
Placement	: Boardge edge

- C24, C25, C27

Capacitance		: 100nF
		Voltage Rating		: 16V or higher

Tolerance		: +- 10%

Minimum package	: 0402

Type			: X7R or X5R

Placement		: Adjacent to VBUS and VDD

- C26, C28

Capacitance		: 4.7uF
		Voltage Rating		: 16V or higher

Tolerance		: +- 10%

Minimum package	: 0603 or 0805

Type			: X7R or X5R

Placement		: In parallel with 100nF caps (C25, C27)

- R14

Resistance		: 10k Ω

Tolerance		: +- 5%

Power Rating		: 1/16W

Minimum package	: 0402

- R15, R16

Resistance		: 100 Ω

Tolerance		: +- 5%

Power Rating		: 1/16W

Minimum package	: 0402

### Power Schema

- Powered by USB VBUS -> Internal 3.3v Regulator

### Signal & Protection

- TVS ESD on D+ and D-

- TXD -> STM32_RX
RXD -> STM32_TX

## 7. EPD Boost Driver

E-Paper Schematic Reference - Google Drive

### Components Specification Requirements

- C21, C22

Capacitance		: 4.7uF
		Voltage Rating		: 50V or higher

Tolerance		: +- 10%

Minimum package	: 0805

Type			: X7R or X5R

- C11~C20

Capacitance		: 1uF
		Voltage Rating		: 16V or higher

Tolerance		: +- 10%

Minimum package	: 0402

Type			: X7R or X5R

Placement		: Adjacent to corresponding FPC connector

- R10

Resistance		: 3 Ω

Tolerance		: +- 5%

Power Rating		: 1/4W

Minimum package	: 0603

- R11

Resistance		: 470k Ω

Tolerance		: +- 5%

Power Rating		: 1/16W

Minimum package	: 0402

- R12

Resistance		: 10k Ω

Tolerance		: +- 5%

Power Rating		: 1/16W

Minimum package	: 0402

- D1, D2, D3
Part Number		: MBR0530
Reverse Voltage	: 30V or higher
Forward Current	: 0.5A
Package		: SOD-123

- Q2 (AO3401A)
MOSFET Type	: P-Channel
Quiescent Current	: 0.1uA ~ 1uA

- Q3
Part Number		: SI1304BDL
Voltage		: 20v to 30v

### Interface Mapping

  - FPC-24

| Pin No. | Datasheet Name1 | Reference Name2 | Reference Connection2 | Current Connection3 |
| --- | --- | --- | --- | --- |
| 1 | NC | HLT_CTL | NC | NC |
| 2 | GDR | GDR | Q3 Gate | Q3 Gate |
| 3 | RESE | RESE | Q3 Source | Q3 Source |
| 4 | NC | VGL | CAP | CAP |
| 5 | VSH2 | VGH | CAP | CAP |
| 6 | TSCL | TSCL | NC | NC |
| 7 | TSDA | TSDA | NC | NC |
| 8 | BS1 | BS | Solder Bridge | Solder Bridge |
| 9 | BUSY | BUSY | GPIO | PA4 (10) |
| 10 | RES# | RST | GPIO | PB1 (15) |
| 11 | D/C# | D/C | GPIO | PB0 (14) |
| 12 | CS# | CS | GPIO | PA6 (12) |
| 13 | SCL | SCLK | GPIO (SPI) | PA5 (11) |
| 14 | SDA | SDI | GPIO (SPI) | PA7 (13) |
| 15 | VDDIO | VDDIO | 3V Power | EPD_VDD |
| 16 | VC1 | VC1 | 3V Power | EPD_VDD |
| 17 | VSS | VSS | GND | GND |
| 18 | VDD | VDD | CAP | CAP |
| 19 | VPP | VPP | CAP | CAP |
| 20 | VSH | VSH | CAP | CAP |
| 21 | VGH | PREVGH | Diode | D3 |
| 22 | VSL | VSL | CAP | CAP |
| 23 | VGL | PREVGL | Diode | D2 |
| 24 | VCOM | VCOM | CAP | CAP |

1 from waveshare 2.13 e-Paper V4 Specification

2 from reference design

3 on this implementation (v1.2)